Getting started with ThingsIO.AI

- Register the account in http://thingsio.ai/#/register.

- Create a project:

- Write the name of the project:

- Now, you will be on the project dashboard of your account. Click on the new project option:

- Click on the add a new device option:

- Enter the device name and click on the create and configure device:

- You will be on your device dashboard:

- Track your device’s special parameters here. This can be set in device configuration (default to null).

- You can see your real time and special parameter’s graph.

- You can see your all data points.

- You can see here your last 5 data points.

- You can see here all the list of created graphs.

- Go in sample device code options and click on the CC3200. You will get the sample code from there and paste into your CCS.

- Click on the “Send trial data” to send a trial data to the server:

- Now, click on the configuration device option.

- You can set the special parameters (this is tracked on the device dashboard) & transform it accordingly and add new parameters (Parameters are updated automatically as you send them from your device).

- Click on the update device option:

- You can also set your device geographical location.

- To create a graph for your device click on the “create graph option”.

- Click on the preview option. You will see your created graph and click save changes.

Platform: Windows

Installation and settings of CC3200 and CCS:

- Follow the installation and settings of CC3200 through this link: https://www.youtube.com/watch?v=xbh9I8waq5g

- Go on project explorer-> file name and right click from mouse and go in properties. Include the file path in ARM compiler-> include options (#include search path)

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\netapps"

"C:\TI\ccsv6\tools\compiler\ti-cgt-arm_5.2.5\include"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\netapps\json"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\driverlib"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\inc"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\example\common"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\simplelink"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\simplelink\source"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\simplelink_extlib\provisioninglib"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\oslib"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\netapps\http\client"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\simplelink\include"

- Include the file path in ARM linker-> file search

->Include library file or command file:

"libc.a"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\driverlib\ccs\Release\driverlib.a"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\simplelink\ccs\NON_OS\simplelink.a"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\oslib\ccs\free_rtos\free_rtos.a"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\netapps\json\ccs\Release\json.a"

“C:\TI\CC3200SDK_1.3.0\cc3200sdk\netapps\http\client\ccs\HTTPClientMinLib\webclient.a"

->Add <dir> to library search path

"${CG_TOOL_ROOT}/lib"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\simplelink\include"

"${CC3200_SDK_ROOT}/netapps/json/ccs/Release/"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\simplelink\ccs\OS"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\netapps\http\client"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\simplelink"

"C:\TI\CC3200SDK_1.3.0\cc3200-sdk\oslib\ccs\free_rtos"

"D:\webclient\HTTPClientMinLib"

Code modification:

- Firstly, mention the host name and url respectively in main.c.

const char *soft_layer = "api.thingsio.ai"

#define CREATEA_SESSION_URI              " /devices/deviceData "

- Change the json data format like:

Change the device ID and slave_id.

{ "device_id": "integer", "dts": "date-time", "slave_id": "integer":, "data": {} }

Example:

#define POST_HEADER                     "{"

#define POST_timestamp                           "\n\"dts\":"

#define POST_DEVICEID                          ",\n\"device_id\":201426,"

#define POST_SLAVE                              "\n\"slave_id\":"

#define POST_HEADER1                    "\n\"data\":"

#define POST_BEGIN                              "{"

//

//

//#define POST_CHUNK_1                            "\n\"device_type\":"

//#define POST_CHUNK_2                            "\n\"e_today\":"

//#define POST_CHUNK_3                            "\n\"e_total\":"

//#define POST_CHUNK_4                            "\n\"h_total\":"

//#define POST_CHUNK_5                            "\n\"running_status\":"

//#define POST_CHUNK_25                            "\n\"qac\":"

//

#define POST_END                                "\n}"

#define POST_TAIL1                              "\n}"

- Change the address and number of slave because here, we are using Modbus protocol. You can remove Modbus when you are using any sensor.

const int Adress[5] = {10001, 10002, 10004, 10006 ,10012}

- Change slave name according to the address

const char*name_list[5] = {"device_type","e_today","e_total", "h_total" , "running_status"}

- Construct_post_buffer()

- Modify the case according to number of slaves :

- start_val = 0;

end_val = 5;

break;

- Change buffer location according to json data:

Example: strcpy(pcBufLocation,POST_HEADER);

pcBufLocation += strlen(POST_HEADER);

strcpy(pcBufLocation,POST_HEADER1);

pcBufLocation += strlen(POST_HEADER1);

- In  HTTPLoginMethod and HTTPPostMethod , please make sure that content type should be “application/json”.

- For wi-fi settings, go in project explorer->your_file->/example/common->common.h

Change SSID_NAME, SECURITY_KEY and SECURITY_TYPE = “SL_SEC_TYPE_WPA” or WPA2

- Plugin the CC3200 through USB cable. Check in device manager the port name and number.

- Check the pin configuration and shorting of pins in CC3200.

- Go in debug option and debug it.

- Open the Hercules or tera-term application to see the output/procedure of the CC3200.

- Go in serial and set the Name: according to your port number, Baud rate: 19200, Data size: 8 and Parity: none and open it.

- Go in CCS and play the code.

- You can see all the response from the server, login details, connection with the wi-fi etc. in Hercules or tera-term.